Supplemental Information

idPAD:  Paper Analytical Device for Presumptive Identification of Illicit Drugs

Table of Contents

S1. idPAD Spotting Guide	2

S2. Blinded Study Sample Preparation	3

S3. Sample Application	10

S4. Light Box	10

S5. idPAD Blinded Study Reader	11

S6. Grading of Blinded Samples	12

S7. Blinded Study idPAD Images	19

S8. T-Testing of Lanes A-C Inverted Blue Color Intensity	49

S9. Heroin Standard idPAD	50

S10. Additional Color Bar Codes	50

S11. Characteristic Curves of Blinded Samples	52

S12. Grading of Adulterant, Cutting Agent and Drug Samples	54

S13. Adulterant, Cutting Agent, and Drug idPAD Images	56

S14. Stability Testing	68

# S1. idPAD Spotting Guide

# S2. Blinded Study Sample Preparation

Identity of street samples was confirmed with FTIR and in some cases, GC-MS. FTIR samples were ground with FTIR-grade KBr powder purchased from Alfa Aesar, 30 Bond Street, Ward Hill, MA 01835. Samples submitted for GC-MS were diluted in sufficient amount of a mixture of dichloromethane:methanol:n-propanol (8:1:1) all ASC-grade, filtered into ThermoScientific 12 x 32 mm (1.8 ml) clear glass vials with screw-top T/RR septa lids and injected with an auto sampler.  The instrument is an Agilent 7890 with a 30-m Rxi-5MS column using Hydrogen as a carrier gas with a LECO Pegasus BT Time-of-Flight mass spectrometer detector.

FTIR scans of the compounds used in the blinded sample study can be found below. These scans were completed in accordance with the Berrien County Crime Lab procedures and identified using their stored and shared libraries.

# S3. Sample Application

Solid sample is applied to the idPAD using a bamboo stir stick. Multiple “swiping” passes across the 12 lanes are made to evenly distribute solid. 10-40mg of solid is recommended depending on the physical characteristics of the powder. The main source of color variability appears to be the reagent application process as these devices are made by hand. If the same volume of liquid reagent is not applied at the exact sample spot of each lane across the card, slight variability can be observed. This is most noticeable when the same reagents are stamped across all 12 lanes but is negligible on idPADs. It should also be noted that the idPAD is not quantitative and serves as a field presumptive test where additional lab analysis would be needed.

A subset of idPAD images is included below where all 12 lanes are of the same color test. As one can see, the color produced is quite consistent.

# S4. Light Box

A lightbox has been fashioned out of a plastic “ammunition box” in which a hole has been cut in the top and LED strip tape applied inside.

# S5. idPAD Blinded Study Reader

# S6. Grading of Blinded Samples

| Sample Code | Drug | Drug Concentration (by Mass) | Cutting Agent (concentration by mass) | Analyst 1 | Analyst 2 | Analyst 3 |
| --- | --- | --- | --- | --- | --- | --- |
| 435 | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| 658 | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| 672 | Methamphetamine | 25% | Dimethyl Sulfone (75%) | Meth | Meth | Meth |
| 736 | None | 0% | Dimethyl Sulfone (100%) | DMS | DMS | Lactose |
| 986 | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |
| 24B | Methamphetamine | 100% | None | Meth | Meth | Meth |
| 2BQ | Crack Cocaine 2 | 100% | None | Crack | Crack | Cocaine |
| 2DJ | Heroin | 100% | None | Heroin | Heroin | Heroin |
| 2HQ | Methamphetamine | 100% | None | Meth | Meth | Meth |
| 2K5 | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| 2N4 | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| 2TN | Heroin | 100% | None | Heroin | Heroin | Heroin |
| 2WN | Methamphetamine | 100% | None | Meth | Meth | Meth |
| 33P | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| 34E | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| 34W | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Cocaine | Cocaine | Crack |
| 3FS | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| 3GM | Heroin | 25% | Lactose (75%) | Heroin | Crack | Heroin |
| 3LS | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| 3PV | Methamphetamine | 100% | None | Meth | Meth | Meth |
| 3R9 | Cocaine HCl | 100% | None | Cocaine | Cocaine | Cocaine |
| 3U4 | Cocaine HCl | 100% | None | Cocaine | Cocaine | Cocaine |
| 44U | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Cocaine | Crack | Crack |
| 44Y | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| 4AZ | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| 4D9 | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |
| 4J9 | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| 4KH | Crack Cocaine 2 | 100% | None | Crack | Crack | Crack |
| 4RD | Cocaine HCl | 100% | None | Cocaine | Cocaine | Cocaine |
| 4TL | Crack Cocaine 2 | 100% | None | Crack | Crack | Cocaine |
| 4TT | Methamphetamine | 100% | None | Meth | Meth | Meth |
| 4V8 | Methamphetamine | 100% | None | Meth | Crack | Meth |
| 4VC | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| 4VQ | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| 4WZ | None | 0% | Dimethyl Sulfone (100%) | DMS | DMS | DMS |
| 52M | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| 57H | None | 0% | Dimethyl Sulfone (100%) | Lactose | Lactose | Lactose |
| 5BS | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| 5C6 | Heroin | 100% | None | Heroin | Heroin | Heroin |
| 5ET | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Crack | Cocaine |
| 5HD | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| 5JK | Cocaine HCl | 100% | None | Cocaine | Cocaine | Crack |
| 5PX | Heroin | 100% | None | Heroin | Heroin | Heroin |
| 5SP | Heroin | 25% | Lactose (75%) | Heroin | Heroin | Heroin |
| 5UJ | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| 5Y7 | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| 67U | None | 0% | Dimethyl Sulfone (100%) | DMS | Lactose | Lactose |
| 6CA | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Crack | Cocaine | Crack |
| 6JZ | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| 6RL | Heroin | 100% | None | Heroin | Heroin | Heroin |
| 6SC | Methamphetamine | 25% | Dimethyl Sulfone (75%) | Meth | Meth | Meth |
| 6SN | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| 7KK | Heroin | 25% | Lactose (75%) | Heroin | Heroin | Heroin |
| 7LK | Heroin | 25% | Lactose (75%) | Heroin | Heroin | Heroin |
| 7NA | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Crack | Crack | Crack |
| 7QD | Crack Cocaine 2 | 100% | None | Crack | Crack | Cocaine |
| 7W4 | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |
| 8F4 | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Cocaine | Meth |
| 8HJ | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Cocaine | Crack | Crack |
| 8KJ | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Cocaine | Heroin | Crack |
| 8RX | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |
| 97U | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |
| 98N | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| 9C9 | Methamphetamine | 25% | Dimethyl Sulfone (75%) | DMS | Meth | Cocaine |
| 9EH | Crack Cocaine 2 | 100% | None | Crack | Crack | Cocaine |
| 9JF | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |
| 9N4 | None | 0% | Dimethyl Sulfone (100%) | DMS | DMS | DMS |
| 9P5 | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| 9WN | Heroin | 50% | Lactose (50%) | Heroin | Crack | Heroin |
| 9XP | Crack Cocaine 2 | 100% | None | Crack | Crack | Cocaine |
| A7Z | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| ADJ | Heroin | 100% | None | Heroin | Heroin | Heroin |
| ARA | None | 0% | Dimethyl Sulfone (100%) | Lactose | Lactose | Lactose |
| AT8 | Methamphetamine | 100% | None | Meth | Meth | Meth |
| AUM | None | 0% | Dimethyl Sulfone (100%) | DMS | DMS | Lactose |
| B3F | Heroin | 25% | Lactose (75%) | Heroin | Crack | Heroin |
| B68 | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| BB9 | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| BDB | Methamphetamine | 25% | Dimethyl Sulfone (75%) | Meth | Meth | Meth |
| BKZ | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| BLK | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Crack | Crack | Crack |
| BNG | Crack Cocaine 2 | 100% | None | Crack | Crack | Cocaine |
| BQ6 | Heroin | 25% | Lactose (75%) | Heroin | Heroin | Heroin |
| BZ9 | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| C8V | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| C97 | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| CNF | Methamphetamine | 25% | Dimethyl Sulfone (75%) | Meth | Meth | Meth |
| CQK | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| CQL | Methamphetamine | 25% | Dimethyl Sulfone (75%) | Meth | Crack | Meth |
| CS9 | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| CSE | Methamphetamine | 25% | Dimethyl Sulfone (75%) | DMS | Crack | Cocaine |
| CUM | Crack Cocaine 2 | 100% | None | Crack | Crack | Cocaine |
| D2P | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| D4G | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| D8H | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| DGU | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| DNZ | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| DPP | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| DQ7 | Cocaine HCl | 100% | None | Cocaine | Cocaine | Cocaine |
| DSW | Cocaine HCl | 100% | None | Cocaine | Cocaine | Cocaine |
| DU9 | Methamphetamine | 100% | None | Meth | Meth | Meth |
| DUG | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| DZB | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |
| E5F | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| E6Y | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| EF8 | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| EHN | Methamphetamine | 25% | Dimethyl Sulfone (75%) | Meth | Meth | Meth |
| EL5 | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| ELJ | Methamphetamine | 25% | Dimethyl Sulfone (75%) | Meth | Meth | Meth |
| ERK | Heroin | 25% | Lactose (75%) | Heroin | Heroin | Heroin |
| EUJ | Heroin | 100% | None | Heroin | Heroin | Heroin |
| EYH | None | 0% | Lactose (100%) | DMS | DMS | DMS |
| F2Q | Cocaine HCl | 100% | None | Cocaine | Cocaine | Crack |
| F4X | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Crack | Crack | Crack |
| FCJ | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| FDB | Methamphetamine | 100% | None | Meth | Meth | Meth |
| FF7 | Heroin | 25% | Lactose (75%) | DMS | DMS | DMS |
| FGB | None | 0% | Dimethyl Sulphone (100%) | Lactose | Lactose | Lactose |
| FJJ | Heroin | 25% | Lactose (75%) | Heroin | Heroin | Heroin |
| FRL | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |
| FRX | Heroin | 25% | Lactose (75%) | Heroin | Heroin | Heroin |
| FV6 | Cocaine HCl | 100% | None | Cocaine | Cocaine | Cocaine |
| FW7 | Crack Cocaine 2 | 100% | None | Crack | Crack | Cocaine |
| FWC | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Crack | Crack | Crack |
| G35 | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| G6Z | Crack Cocaine 2 | 100% | None | Crack | Crack | Cocaine |
| GCT | Methamphetamine | 25% | Dimethyl Sulfone (75%) | Meth | Meth | Meth |
| GGD | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |
| GKC | None | 0% | Dimethyl Sulfone (100%) | DMS | DMS | Lactose |
| GLK | Crack Cocaine 2 | 100% | None | Crack | Crack | Crack |
| GLV | Heroin | 25% | Lactose (75%) | Heroin | Heroin | Heroin |
| GPN | Methamphetamine | 100% | None | Meth | Meth | Meth |
| GTQ | Methamphetamine | 25% | Dimethyl Sulfone (75%) | Meth | Meth | Meth |
| GX3 | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| H2C | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Crack | Crack | Crack |
| H4Y | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| H8G | Methamphetamine | 100% | None | Meth | Meth | Meth |
| HDH | Methamphetamine | 100% | None | Meth | Meth | Meth |
| HH9 | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| HJJ | Heroin | 100% | None | Heroin | Heroin | Heroin |
| J43 | Heroin | 100% | None | Heroin | Heroin | Heroin |
| J95 | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Crack | Cocaine | Crack |
| JAY | Methamphetamine | 25% | Dimethyl Sulfone (75%) | Meth | Meth | Meth |
| JEM | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| JG2 | None | 0% | Lactose (100%) | DMS | Lactose | Lactose |
| JHJ | None | 0% | Dimethyl Sulfone (100%) | DMS | Lactose | Lactose |
| JKL | None | 0% | Dimethyl Sulfone (100%) | Lactose | Lactose | Lactose |
| JTB | Heroin | 25% | Lactose (75%) | Heroin | Crack | Heroin |
| K67 | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| K8L | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |
| K8S | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| K96 | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| KFW | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| KNX | Methamphetamine | 100% | None | Meth | Meth | Meth |
| KPX | Heroin | 25% | Lactose (75%) | Heroin | Heroin | Heroin |
| KQD | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| KSP | Cocaine HCl | 100% | None | Cocaine | Cocaine | Crack |
| L3T | Heroin | 50% | Lactose (50%) | Heroin | Crack | Heroin |
| L4A | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| LBN | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| LGY | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| LKL | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| LTF | Methamphetamine | 25% | Dimethyl Sulfone (75%) | Meth | Meth | Meth |
| LUL | Cocaine HCl | 50% | Lactose (50%) | Crack | Cocaine | Crack |
| LVA | Cocaine HCl | 100% | None | Cocaine | Cocaine | Cocaine |
| LWK | Cocaine HCl | 100% | None | Cocaine | Heroin | Cocaine |
| M6P | Cocaine HCl | 100% | None | Cocaine | Cocaine | Cocaine |
| MAD | Heroin | 25% | Lactose (75%) | Heroin | Heroin | Heroin |
| MB5 | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| MBA | None | 0% | Lactose (100%) | Lactose | DMS | Lactose |
| MCP | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| MGF | Crack Cocaine 2 | 100% | None | Crack | Cocaine | Cocaine |
| MJ4 | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| MTJ | Heroin | 25% | Lactose (75%) | Heroin | Heroin | Heroin |
| MUB | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| N2D | Heroin | 100% | None | Heroin | Heroin | Heroin |
| NA4 | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| NBR | None | 0% | Dimethyl Sulfone (100%) | DMS | DMS | DMS |
| NV3 | Heroin | 25% | Lactose (75%) | Heroin | Heroin | Heroin |
| NVV | Cocaine HCl | 100% | None | Cocaine | Cocaine | Cocaine |
| OOO | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| P3H | Cocaine HCl | 100% | None | Cocaine | Cocaine | Cocaine |
| P5D | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| P5R | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| P6V | None | 0% | Dimethyl Sulfone (100%) | Lactose | Lactose | Lactose |
| P8S | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| PE6 | Heroin | 100% | None | Heroin | Heroin | Heroin |
| PJW | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| PNN | Crack Cocaine 2 | 100% | None | Crack | Crack | Crack |
| PPV | Methamphetamine | 100% | None | Meth | Meth | Meth |
| PUH | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| PZT | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| Q69 | Crack Cocaine 2 | 100% | None | Crack | Crack | Cocaine |
| Q6B | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Crack | Crack | Crack |
| QAE | Cocaine HCl | 100% | None | Cocaine | Cocaine | Cocaine |
| QFH | Heroin | 25% | Lactose (75%) | Heroin | Heroin | Heroin |
| QMR | Crack Cocaine 2 | 100% | None | Crack | Crack | Cocaine |
| QPZ | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| QTE | Methamphetamine | 25% | Dimethyl Sulfone (75%) | DMS | Meth | DMS |
| QUC | Crack Cocaine 2 | 100% | None | Crack | Crack | Cocaine |
| QVC | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| R66 | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| RKD | Methamphetamine | 25% | Dimethyl Sulfone (75%) | Meth | Meth | Meth |
| RNR | None | 0% | Lactose (100%) | DMS | Lactose | Lactose |
| RPX | Heroin | 25% | Lactose (75%) | Heroin | Crack | Heroin |
| RW8 | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| S6H | Methamphetamine | 100% | None | Crack | Meth | Meth |
| SEB | Cocaine HCl | 100% | None | Cocaine | Cocaine | Cocaine |
| SFR | Cocaine HCl | 100% | None | Cocaine | Cocaine | Cocaine |
| SRG | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Crack |
| SSB | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Crack | Cocaine | Crack |
| SWR | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| T2K | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| TGP | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| TNL | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Cocaine | Crack | Crack |
| TTQ | Methamphetamine | 100% | None | Meth | Meth | Meth |
| TU4 | None | 0% | Dimethyl Sulfone (100%) | Lactose | Lactose | Lactose |
| UEX | Crack Cocaine 2 | 100% | None | Crack | Crack | Cocaine |
| UFL | Heroin | 100% | None | Heroin | Heroin | Heroin |
| UJ5 | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| UJG | Heroin | 100% | None | Heroin | Heroin | Heroin |
| ULN | Methamphetamine | 100% | None | Meth | Meth | Meth |
| UM8 | Crack Cocaine 2 | 100% | None | Crack | Heroin | Cocaine |
| UNE | Crack Cocaine 2 | 100% | None | Crack | Crack | Crack |
| UR9 | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| URU | Heroin | 25% | Lactose (75%) | Heroin | Crack | Heroin |
| UZW | None | 0% | Lactose (100%) | Lactose | Lactose | DMS |
| V25 | Cocaine HCl | 100% | None | Cocaine | Cocaine | Cocaine |
| V9G | None | 0% | Dimethyl Sulfone (100%) | DMS | Lactose | DMS |
| VC3 | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| VEL | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| VKM | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| VKR | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| VQS | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Crack | Crack | Crack |
| VU6 | Methamphetamine | 50% | Dimethyl Sulfone (50%) | Meth | Meth | Meth |
| VXT | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Cocaine | Cocaine | Crack |
| VYN | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Crack | Crack | Crack |
| W48 | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |
| W9F | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |
| WA9 | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Crack | Crack | Crack |
| WAN | Heroin | 100% | None | Heroin | Heroin | Heroin |
| WHH | Crack Cocaine 2 | 100% | None | Crack | Crack | Crack |
| WL7 | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Crack |
| WNL | Heroin | 25% | Lactose (75%) | Heroin | Heroin | Heroin |
| WPG | Methamphetamine | 100% | None | Meth | Cocaine | Meth |
| WRB | Methamphetamine | 100% | None | Meth | Meth | Meth |
| WTZ | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |
| WVF | Heroin | 50% | Lactose (50%) | Heroin | Heroin | Heroin |
| WWB | Methamphetamine | 25% | Dimethyl Sulfone (75%) | Meth | Meth | Meth |
| X8B | None | 0% | Dimethyl Sulfone (100%) | DMS | DMS | Lactose |
| X93 | Heroin | 100% | None | Heroin | Heroin | Heroin |
| XAH | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Crack | Crack | Crack |
| XBH | None | 0% | Lactose (100%) | DMS | Lactose | Lactose |
| XBZ | Methamphetamine | 25% | Dimethyl Sulfone (75%) | Meth | Meth | Meth |
| XDX | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| XET | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| XHM | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |
| XRP | Cocaine HCl | 100% | None | Cocaine | Cocaine | Cocaine |
| XS9 | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |
| XSQ | Cocaine HCl | 100% | None | Cocaine | Cocaine | Cocaine |
| XVT | None | 0% | Dimethyl Sulfone (100%) | Lactose | DMS | Lactose |
| XWN | Methamphetamine | 25% | Dimethyl Sulfone (75%) | Meth | Meth | Meth |
| Y6J | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| YCR | Heroin | 100% | None | Heroin | Heroin | Heroin |
| YG4 | Heroin | 100% | None | Heroin | Heroin | Heroin |
| YL7 | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |
| YTB | Crack Cocaine 1 | 100% | Cut with Nicotinamide | Cocaine | Crack | Crack |
| YUD | None | 0% | Lactose (100%) | Lactose | Lactose | Lactose |
| Z5B | Heroin | 100% | None | Heroin | Crack | Heroin |
| Z8J | None | 0% | Dimethyl Sulfone (100%) | DMS | DMS | DMS |
| ZHM | Heroin | 100% | None | Heroin | Heroin | Heroin |
| ZJS | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |
| ZKS | Cocaine HCl | 25% | Lactose (75%) | Cocaine | Cocaine | Cocaine |
| ZNX | Methamphetamine | 100% | None | Meth | Meth | Meth |
| ZR4 | Heroin | 100% | None | Heroin | Heroin | Heroin |
| ZS2 | None | 0% | Dimethyl Sulfone (100%) | DMS | Lactose | Lactose |
| ZSC | Methamphetamine | 25% | Dimethyl Sulfone (75%) | Meth | Meth | Meth |
| ZZ4 | Cocaine HCl | 50% | Lactose (50%) | Cocaine | Cocaine | Cocaine |

# S7. Blinded Study idPAD Images

# S8. T-Testing of Lanes A-C Inverted Blue Color Intensity

n=46 for each Cocaine and Heroin

| Lane | Inverted Mean (x) Pixel Value | Standard Deviation () | P Value (Two-Tailed) |
| --- | --- | --- | --- |
| Lane A Cocaine | 0.856 | 0.028 | 2.921 x 10-15 |
| Lane A Heroin | 1.485 | 0.115 |  |
| Lane B Cocaine | 0.739 | 0.020 | 1.469 x 10-21 |
| Lane B Heroin | 1.460 | 0.056 |  |
| Lane C Cocaine | 0.665 | 0.039 | 1.072 x 10-20 |
| Lane C Heroin | 1.368 | 0.044 |  |
| Lanes A-C Cocaine | 0.771 | 0.029 | 2.437 x 10-53 |
| Lanes A-C Heroin | 1.440 | 0.074 |  |

# S9. Heroin Standard idPAD

20uL of a 1mg/mL Heroin in methanol standard (Cerilliant, Lot: FE06151603, Exp: 11/2021) was applied to the idPAD. Lane D (right) and Lane E (left) test for phenols. Both lanes had faint reactivity by turning orange/red when exposed to the heroin standard.

Blank idPAD 	       Heroin Std. idPAD

D      E

D      E

D       E

D       E

# S10. Additional Color Bar Codes

# S11. Characteristic Curves of Blinded Samples

The characteristic curves below display the blinded sample identifications split up by drug and researcher. For each drug and dilution (100% heroin, 50% heroin/50% lactose, etc.) a correct sample ID=1, a wrong ID=0, and a close ID= 0.5 (only done for Cocaine HCl vs Crack Cocaine). The results were averaged and plotted on the y-axis with the x-axis indicating increasing concentration/percentage of controlled substance. These plots clearly show that the majority of false negative identifications occurred when the sample contained only 25% drug. Analysts 1 and 2 were expert readers with Analyst 3 as the novice.

# S12. Grading of Adulterant, Cutting Agent and Drug Samples

| Sample Code | Actual Identity | Analyst 1: Drug Present? | Analyst 1: If so, which? | Analyst 2: Drug Present? | Analyst 2: If so, which? |
| --- | --- | --- | --- | --- | --- |
| 7MB | 1,2,4,5-benzenetetra | no |  | No |  |
| 4BJ | 2-aminothiazole | no |  | No |  |
| PTW | 4-4-Dipyridil | no |  | No |  |
| 3UP | 4-aminobenzonitrile | no |  | No |  |
| 9ZG | 7,7,8,8-Tetracyanoquinodimethane | no |  | No |  |
| 2RQ | Acetaminophen | no |  | No |  |
| NTA | Acetaminophen Hydrocodone | no |  | Yes | Meth |
| 7JW | Alizarian Compuxone | no |  | No |  |
| 2UW | Azithromycin | no |  | No |  |
| N7D | Carbonyldimidazole | no |  | No |  |
| HJM | Carboxybenzaldehyde | no |  | No |  |
| 8EB | Chloramphenicol | no |  | No |  |
| UVW | Chloranil | no |  | No |  |
| EP6 | Chloranilic Acid | no |  | No |  |
| YLK | Ciprofloxacin | no |  | No |  |
| KVZ | Clonazepam | no |  | No |  |
| 39R | Cocaine | yes | Cocaine | Yes | Cocaine |
| 9PE | Cocaine | yes | Cocaine | Yes | Cocaine |
| AWM | Cocaine | yes | Cocaine | Yes | Cocaine |
| DPP | Cocaine | yes | Cocaine | Yes | Cocaine |
| EWY | Cocaine | yes | Cocaine | Yes | Cocaine |
| NCQ | Cocaine | yes | Cocaine | Yes | Cocaine |
| RYD | Cocaine | yes | Cocaine | Yes | Cocaine |
| T9H | Cocaine | yes | Cocaine | Yes | Cocaine |
| WAQ | Cocaine | yes | Cocaine | Yes | Cocaine |
| YJN | Cocaine | yes | Cocaine | Yes | Cocaine |
| RDG | D-Amphetamine | no |  | No |  |
| QQ8 | Dexamethason | no |  | No |  |
| W4U | Dichlorophenolindophenol | no |  | No |  |
| DQH | diethylphenyl benzamide | no |  | No |  |
| TA3 | DimethylaminoCinnamaldehyde | no |  | No |  |
| YA7 | Enalapril | no |  | No |  |
| LB8 | Epinephrine | no |  | No |  |
| 7PP | Erythromycin | no |  | No |  |
| PBQ | Ethambutol | no |  | No |  |
| G6V | Furosemide | no |  | No |  |
| GG7 | Gabapentin | no |  | No |  |
| GPC | Glycine | no |  | No |  |
| 473 | Heroin | yes | Heroin | Yes | Heroin |
| 4P9 | Heroin | yes | Heroin | Yes | Heroin |
| 7EA | Heroin | yes | Heroin | Yes | Heroin |
| 8KD | Heroin | yes | Heroin | Yes | Heroin |
| AEM | Heroin | yes | Heroin | Yes | Heroin |
| BWR | Heroin | yes | Heroin | Yes | Heroin |
| FXN | Heroin | yes | Heroin | Yes | Heroin |
| JYX | Heroin | yes | Heroin | Yes | Heroin |
| LFY | Heroin | yes | Heroin | Yes | Heroin |
| PHT | Heroin | yes | Heroin | Yes | Heroin |
| K9X | Hydantoin | no |  | No |  |
| 7WY | Hydroquinone | no |  | No |  |
| 7RW | Imidazole | no |  | No |  |
| 6BW | Isatin | no |  | No |  |
| 4V8 | Mebendazole | no |  | No |  |
| 49X | Meth | yes | Meth | Yes | Meth |
| 84C | Meth | yes | Meth | Yes | Meth |
| BHB | Meth | yes | Meth | Yes | Meth |
| CBB | Meth | yes | Meth | Yes | Meth |
| JA6 | Meth | yes | Meth | Yes | Meth |
| PBV | Meth | yes | Meth | Yes | Meth |
| QYP | Meth | yes | Meth | Yes | Meth |
| UUK | Meth | yes | Meth | Yes | Meth |
| V9P | Meth | yes | Meth | Yes | Meth |
| WY9 | Meth | yes | Meth | Yes | Meth |
| YUF | Meth | yes | Meth | Yes | Meth |
| 299 | Metronidazole | no |  | No |  |
| 8QC | Morphine | no |  | No |  |
| D9Z | Ninhydrin | no |  | No |  |
| WRV | NQS | no |  | No |  |
| NLV | O-Phenonthymine | no |  | No |  |
| QM2 | Orcinol | no |  | No |  |
| D6W | Penicillin G | no |  | No |  |
| CN9 | Percocet | no |  | Yes | Meth |
| U7P | Phenoxazine | no |  | No |  |
| WD7 | Phthalamide Potassium | no |  | No |  |
| 2OO | Procaine | no |  | No |  |
| R7X | Pyrazinamide | no |  | No |  |
| VYF | Pyrazole | no |  | No |  |
| V2D | Quinine | no |  | No |  |
| XPA | Quinine Sulfate | no |  | No |  |
| DYC | Rizatriptan Benzoagte | no |  | No |  |
| MAL | Safranin O | no |  | No |  |
| CGZ | Sodium Citrate Dihydrate | no |  | No |  |
| LHQ | Sodium Tetraphenyl Boroate | no |  | No |  |
| QQZ | Sulfadozine | no |  | No |  |
| KDC | Sulfamethoxazole | no |  | No |  |
| 47Z | Sulfanilamine | no |  | No |  |
| AVP | Syringaldazin | no |  | No |  |
| BXF | Tetrahydroxy-1,4-quinone hydrate | no |  | No |  |
| 92V | Tetraphyltgium Disulfide | no |  | No |  |
| XPU | Tramadol | no |  | Yes | Cocaine |
| BG3 | Triphenylphosphine | no |  | No |  |
| BL4 | Uridine | no |  | No |  |
| 8LK | Vanillin | no |  | No |  |
| KDB | Water | no |  | No |  |
| TDV | Water | no |  | No |  |
| LQD | Xanax | no |  | No |  |

# S13. Adulterant, Cutting Agent, and Drug idPAD Images

# S14. Stability Testing

| Lane | Chemical Test | Analyte Used | 1 Week | 1 Month | 3 Months |
| --- | --- | --- | --- | --- | --- |
| A | Acidic Co(SCN)2 | Ciprofloxacin | Hot: PASS | Hot: PASS | Hot: PASS |
|  |  |  | Lab Temp: PASS | Lab Temp: PASS | Lab Temp: PASS |
|  |  |  | Cold: PASS | Cold: PASS | Cold: PASS |
| B | Neutral Co(SCN)2 | Diethylcarbamazine Citrate | Hot: PASS | Hot: PASS | Hot: PASS |
|  |  |  | Lab Temp: PASS | Lab Temp: PASS | Lab Temp: PASS |
|  |  |  | Cold: PASS | Cold: PASS | Cold: PASS |
| C | Basic Co(SCN)2 | Ciprofloxacin | Hot: PASS | Hot: FAIL | Hot: FAIL |
|  |  |  | Lab Temp: PASS | Lab Temp: PASS | Lab Temp: PASS |
|  |  |  | Cold: PASS | Cold: PASS | Cold: PASS |
| D | P-Nitroaniline | 50% Amoxicillin/50% Starch | Hot: PASS | Hot: PASS | Hot: PASS |
|  |  |  | Lab Temp: PASS | Lab Temp: PASS | Lab Temp: PASS |
|  |  |  | Cold: PASS | Cold: PASS | Cold: PASS |
| E | Sulfanilamide | 50% Amoxicillin/50% Starch | Hot: PASS | Hot: PASS | Hot: PASS |
|  |  |  | Lab Temp: PASS | Lab Temp: PASS | Lab Temp: PASS |
|  |  |  | Cold: PASS | Cold: PASS | Cold: PASS |
| F | Naphthoquinone Sulfonate | Ciprofloxacin | Hot: PASS | Hot: PASS | Hot: PASS |
|  |  |  | Lab Temp: PASS | Lab Temp: PASS | Lab Temp: PASS |
|  |  |  | Cold: PASS | Cold: PASS | Cold: PASS |
| G | Starch | 50% Amoxicillin/50% Starch | Hot: PASS | Hot: PASS | Hot: PASS |
|  |  |  | Lab Temp: PASS | Lab Temp: PASS | Lab Temp: PASS |
|  |  |  | Cold: PASS | Cold: PASS | Cold: PASS |
| H | Ethambutol | 50% Amoxicillin/50% Starch | Hot: PASS | Hot: FAIL | Hot: FAIL |
|  |  |  | Lab Temp: PASS | Lab Temp: PASS | Lab Temp: PASS |
|  |  |  | Cold: PASS | Cold: PASS | Cold: PASS |
| I | FeCl3 | Epinephrine | Hot: PASS | Hot: PASS | Hot: PASS |
|  |  |  | Lab Temp: PASS | Lab Temp: PASS | Lab Temp: PASS |
|  |  |  | Cold: PASS | Cold: PASS | Cold: PASS |
| J | Ninhydrin | 50% Amoxicillin/50% Starch | Hot: PASS | Hot: PASS | Hot: PASS |
|  |  |  | Lab Temp: PASS | Lab Temp: PASS | Lab Temp: PASS |
|  |  |  | Cold: PASS | Cold: PASS | Cold: PASS |
| K | Dimethylamino-cinnamaldehyde | Indole | Hot: PASS | Hot: PASS | Hot: PASS |
|  |  |  | Lab Temp: PASS | Lab Temp: PASS | Lab Temp: PASS |
|  |  |  | Cold: PASS | Cold: PASS | Cold: PASS |
| L | Timer | Water | Hot: PASS | Hot: PASS | Hot: PASS |
|  |  |  | Lab Temp: PASS | Lab Temp: PASS | Lab Temp: PASS |
|  |  |  | Cold: PASS | Cold: PASS | Cold: PASS |